# FRONT MATTER

## TITLE PAGE

# ROADWISE AI - EDGE-AI BASED REAL-TIME POTHOLE DETECTION AND CLASSIFICATION USING SMARTPHONE ACCELEROMETER AND SPECTRAL ANALYSIS

### A MAJOR PROJECT REPORT SUBMITTED

### IN PARTIAL FULFILLMENT OF THE REQUIREMENTS

### FOR THE AWARD OF DEGREE OF

## BACHELOR OF TECHNOLOGY

### in

### Computer Science and Engineering

### SUBMITTED BY:

Simriti Kak (Roll No: 2022a1r004)
Bhuvan Sharma (Roll No: 2022a1r003)
Pratham Seth (Roll No: 2022a1r037)

### UNDER THE SUPERVISION OF:

[Supervisor Name]
[Academic Designation], Department of Computer Science and Engineering

### Department of Computer Science and Engineering

## Model Institute of Engineering and Technology (Autonomous)

### Kot Bhalwal, Jammu, India

### (NAAC “A” Grade Accredited)

### May, 2026

## CANDIDATE’S DECLARATION

I/We, Simriti Kak (Roll No: 2022a1r004), Bhuvan Sharma (Roll No: 2022a1r003), and Pratham Seth (Roll No: 2022a1r037), hereby declare that the work being presented in the major project entitled, “ROADWISE AI - EDGE-AI BASED REAL-TIME POTHOLE DETECTION AND CLASSIFICATION USING SMARTPHONE ACCELEROMETER AND SPECTRAL ANALYSIS” in partial fulfillment of the requirements for the award of the degree of B.Tech., submitted in the Department of Computer Science and Engineering, Model Institute of Engineering and Technology (Autonomous), Jammu, is an authentic record of our own work carried out under the supervision of [Supervisor Name] ([Academic Designation], Department of Computer Science and Engineering, MIET, Jammu).

The matter presented in this report has not been submitted in this or any other University/Institute for the award of the B.Tech. degree.

Simriti Kak
Bhuvan Sharma
Pratham Seth

Dated: May ____, 2026
Department: Computer Science and Engineering
Institute: Model Institute of Engineering and Technology (Autonomous), Jammu

## CERTIFICATE

Certified that this major project report entitled “ROADWISE AI - EDGE-AI BASED REAL-TIME POTHOLE DETECTION AND CLASSIFICATION USING SMARTPHONE ACCELEROMETER AND SPECTRAL ANALYSIS” is the bonafide work of Simriti Kak (Roll No: 2022a1r004), Bhuvan Sharma (Roll No: 2022a1r003), and Pratham Seth (Roll No: 2022a1r037) of 8th Semester, Computer Science and Engineering, Model Institute of Engineering and Technology (Autonomous), Jammu, who carried out the major project work under our supervision during February 2026 – May 2026.

[Supervisor Name]
Supervisor, Academic Designation
CSE Department, MIET Jammu

This is to certify that the above statement is correct to the best of our knowledge.

Prof. ABC
HoD / Head PRC Committee
CSE Department, MIET Jammu

## ABSTRACT

Poor road infrastructure and undetected surface hazards, such as potholes and improper speed breakers, pose severe threats to commuter safety, cause vehicle wear, and incur major economic costs globally. Conventional municipal monitoring relies on manual inspections or expensive scanning vehicles, leading to massive delays in maintenance. This project introduces RoadWise AI, an automated, zero-hardware, and privacy-preserving Edge-AI ecosystem that transforms standard passenger vehicles into mobile road-quality probes using commercial smartphones.

Operating as an Android background service (DriveGuardService), the system leverages low-power activity recognition to monitor driving behavior dynamically. Raw tri-axial accelerometer and gravity data are collected at 20Hz and mathematically processed through a rotation-invariant gravity projection engine (Earth Z-axis alignment). To eliminate false positives, the system filters physical impacts using a peak G-force threshold of 1.2 m/s² and applies a sliding 2-second window (40 samples, 50% overlap) with standard mean-centered high-pass normalization.

A 12-feature mathematical vector—comprising vertical and lateral statistical features such as root mean square (RMS), energy, Pearson skewness, excess kurtosis, and impact ratio—is extracted and fed into a local machine learning classifier. Running entirely on-device via ONNX Runtime Mobile, the model achieves sub-100ms inference to categorize road features into smooth paths, speed bumps, or potholes with a high-confidence threshold (>= 70%). Lockout durations (1000ms) and speed-dependent buffering in a dedicated DetectionManager prevent duplicated or erroneous low-speed records.

Detections are cached locally and synchronized silently to Google Firebase Firestore under a shared "potholes" collection when authenticated via Firebase Auth. The spatial intelligence layer processes crowdsourced logs to render zoom-aware mapping overlays on OpenStreetMap (osmdroid). At high zoom levels, safety-critical hazards appear as intensity-weighted radial heatmap blobs, while low zoom levels render an A-to-F road quality graded grid on a 100m resolution.

To empower commuters, a real-time hazard avoidance routing module encloses severe pothole coordinates within 20-meter polygon vectors and queries the OpenRouteService API with these avoid zones, generating optimal driving paths that navigate away from distressed roads. Municipal engineers are provided with a dedicated React web dashboard (admin-web) to inspect crowdsourced heatmaps, view historical summaries, and update pothole resolution status in Firestore, establishing an efficient, data-driven bridge between citizen sensing and administrative road maintenance.

Keywords: Edge-AI, ONNX Runtime Mobile, Pothole Detection, Inertial Sensing, Crowd-Sensing, Smart Cities, Path Avoidance Navigation. # CHAPTER 1 # INTRODUCTION

## 1.1 Background

The rapid expansion of urban centers, coupled with the exponential growth of national highway networks, forms the fundamental pillar of global economic growth and societal connectivity. Transportation infrastructure facilitates the seamless transit of goods, services, and human capital, directly influencing the Gross Domestic Product (GDP) of nations [1]. However, maintaining the physical integrity of this vast and complex infrastructure remains one of the most critical, resource-intensive, and persistent challenges faced by municipal bodies, civil engineers, and highway authorities worldwide, particularly in developing economies such as India.

Road surface anomalies—encompassing a wide spectrum of distresses such as potholes, localized depressions, structural alligator cracking, rutting, and improperly constructed, non-standard speed breakers—are inevitable consequences of infrastructure aging. These surfaces rapidly deteriorate under the compounding effects of heavy vehicular loads, inadequate drainage systems, and extreme weather cycles, including seasonal monsoon rains and thermal expansion. When left unaddressed, these surface distresses compromise vehicular safety, escalate transit times, reduce fuel efficiency, and cause severe mechanical wear and tear to passenger vehicles [2]. The cascading economic and social impact of degraded roads is immense, necessitating a paradigm shift from reactive to proactive maintenance strategies.

Historically, road quality assessment and anomaly monitoring have relied on two primary methodologies: manual visual inspection patrols and high-end specialized profiling vehicles. Manual surveys require field inspectors to physically travel along roadways, visually identify distresses, and log coordinates using rudimentary tools. This traditional workflow is highly subjective, labor-intensive, hazardous to the inspectors, and logistically impossible to execute at scale across extensive rural and urban road networks. The frequency of such inspections is entirely inadequate for capturing the rapid degradation that occurs during monsoon seasons.

Conversely, specialized inspection vehicles equipped with high-resolution laser scanners (LiDAR), suspension-mounted distance sensors, and high-speed photometric cameras offer high precision and objective profiling [3]. However, these vehicles are exceptionally expensive to procure, operate, and maintain. As a result, they are typically deployed only on critical national highways or toll roads, leaving secondary urban streets and rural networks unmonitored. Consequently, municipal road quality databases are rarely updated in real-time, resulting in a reactive maintenance model where critical hazards remain unreported for months until they cause a severe incident or a formal citizen complaint is lodged [4].

With the ubiquitous adoption of smartphones over the last decade, modern mobile devices have evolved into highly capable, compact sensing platforms. Equipped with high-frequency micro-electromechanical systems (MEMS) such as tri-axial accelerometers, gyroscopes, magnetometers, and Global Positioning System (GPS) receivers, a standard smartphone possesses the baseline hardware necessary to capture physical kinetic forces with high fidelity [5]. When placed inside a moving vehicle, the smartphone experiences kinetic shocks and vibrations that directly correlate to the structural profile of the road surface beneath the tires. By utilizing advanced inertial sensing algorithms and machine learning, it is possible to transform everyday passenger vehicles into a distributed network of mobile probes. This concept, known as mobile crowdsourced sensing (MCS), democratizes road infrastructure monitoring by turning ordinary commuters into passive, continuous contributors to a collective, real-time spatial road-health database [6].

## 1.2 Motivation

The core motivation driving the conceptualization, design, and development of RoadWise AI is deeply rooted in a critical, data-backed public safety emergency. According to the Ministry of Road Transport and Highways (MoRTH), road accidents claim hundreds of thousands of lives in India annually, with pothole-related incidents accounting for a disproportionately high share of severe injuries and fatalities [7]. The gravity of this crisis is illustrated by several alarming indicators that necessitate urgent technological intervention:

- Surging Anomaly Fatalities: Parliament data reveals a staggering 53% rise in pothole-related fatalities over a four-year period. This tragic statistic underscores the systemic failure of traditional, slow-moving municipal detection and repair cycles, which are unable to keep pace with the rate of road degradation.

- Severe Economic Impact: Road crashes and infrastructure damage cost India approximately 3.14% of its annual Gross Domestic Product (GDP). This represents a massive misallocation of financial resources that are redirected from sustainable development towards emergency healthcare and vehicle repairs [8].

- Prolonged Repair Latency: On average, a dangerous road anomaly remains open and unrepaired for three to six months due to bureaucratic reporting delays, complex tender processes, and a lack of centralized tracking. During this extended latency period, millions of daily commuters are exposed to severe risk.

Furthermore, existing digital citizen reporting applications require drivers or passengers to manually photograph and submit reports. This presents a severe safety hazard, as drivers are distracted while trying to document defects, and passengers may miss anomalies entirely, especially at night or in adverse weather. The research is motivated by the pressing need for a completely automated, zero-hardware, and privacy-preserving Edge-AI system [9]. By running the machine learning model directly on the user’s smartphone, the application can passively classify road distresses without requiring the upload of raw, private, or continuous sensor feeds to the cloud. This decentralized approach ensures the system operates autonomously, even in remote or rural areas with poor network coverage, safeguarding user privacy while contributing to public safety.

Beyond individual commuter safety, there is a clear, unfulfilled demand from municipal administrative bodies for structured, spatial analytics. Instead of scattered, unstructured citizen emails, social media complaints, or phone logs, public works authorities need an intelligent, centralized dashboard. Integrating a crowdsourcing platform with an automated grading engine allows municipal corporations to visualize vast road networks classified into standard grades (A through F) at a highly granular 100-meter resolution. This enables data-driven, predictive road maintenance, transforming municipal workflows from a reactive “complaint-and-patch” model to a proactive, optimal allocation of public works budgets and resources [10].

## 1.3 Research Objectives

To directly address the fundamental limitations of existing manual inspection frameworks and high-cost profiling vehicles, this major project sets out to design, implement, and rigorously validate RoadWise AI—a comprehensive Edge-AI ecosystem. The technical and operational objectives of this research project are defined as follows:

- Develop a Robust, Background-Stable Mobile Sensing Pipeline: Design a native Android application that runs a highly optimized background service (DriveGuardService). Utilizing low-power activity recognition APIs, the service must automatically commence sensor monitoring when driving is detected and gracefully pause when the vehicle is stationary, thereby minimizing battery drain and maximizing background persistence.

- Formulate an Advanced Rotation-Invariant Inertial Preprocessing Engine: Implement robust mathematical algorithms to isolate the true vertical motion of the vehicle by projecting raw 3D linear accelerometer signals onto the gravity vector (Earth Z-axis). This ensures consistent, high-fidelity detection accuracy regardless of the smartphone’s mounting angle, orientation, or physical placement inside the car cabin.

- Implement On-Device Edge-AI Classification using ONNX Mobile: Extract a standardized, highly descriptive 12-feature statistical vector from windowed vibration signals. Run local, real-time machine learning inference using highly optimized ONNX Runtime Mobile binaries, successfully classifying kinetic events into Potholes, Speed Bumps, or Smooth roads at sub-100ms latencies on standard consumer hardware.

- Create a Dual-Layer Spatial Road Quality Mapping System: Aggregate verified hazard coordinates and construct an interactive, zoom-aware map overlay. The system will display radial heatmap indicators of varying intensities at high zoom levels, and automatically divide the road network into a graded 100-meter grid (Grades A to F) at lower zoom levels based on a custom intensity-weighted penalty engine.

- Design a Real-Time Hazard Avoidance Routing Engine: Integrate the OpenRouteService API to calculate optimal driving routes that dynamically bypass dangerous road segments. This involves enclosing detected high-severity potholes within 20-meter spatial avoidance polygons and generating detour paths that prioritize commuter safety over absolute shortest distance.

- Construct an Administrative Web Dashboard for Municipal Governance: Develop a comprehensive companion web interface (admin-web) linked to a highly scalable Firebase Firestore cloud backend. This dashboard will enable public works departments to monitor crowdsourced road-health heatmaps, audit raw hazard logs, and formally mark road sections as resolved post-repair, establishing a closed-loop maintenance lifecycle.

## 1.4 Scope of the Project

The scope of RoadWise AI is defined carefully within the boundaries of practical smartphone sensing limits, processing constraints, and real-world deployment parameters to ensure a viable, scalable product:

- Hardware Boundaries: The system operates strictly using standard, commercially available Android smartphones equipped with basic MEMS accelerometers, hardware gravity sensors, and GPS receivers. No external On-Board Diagnostics (OBD-II) adapters, chassis-mounted suspension sensors, or dedicated hardware cameras are required, ensuring maximum democratization.

- Environmental Scope: Sensor data collection and anomaly classification are designed to operate robustly under all weather and lighting conditions. This includes night driving, heavy rain, fog, or flooded roads, because the system relies entirely on inertial physics and suspension dynamics rather than visual computer vision models, which are highly prone to failure in low visibility [11].

- Vehicular Scope: The vibration signature preprocessing algorithms and machine learning thresholds are optimized for standard passenger vehicles, including hatchbacks, sedans, and light SUVs. Heavy industrial trucks, commercial buses with pneumatic suspensions, or rigid-frame two-wheelers possess significantly different suspension transfer functions and fall outside the primary calibration limits of the current model.

- Routing and Navigation: The dynamic hazard avoidance navigation module utilizes open-source spatial maps (OpenStreetMap via osmdroid) and the OpenRouteService API, targeting regions mapped within their global routing database, with a primary focus on Indian urban arteries and national highway corridors.

- Administrative Interface: The administrative control dashboard is designed primarily for desktop web browsers. It utilizes secure Firebase Authentication and Role-Based Access Control (RBAC) to ensure that only authorized municipal employees or system administrators can modify records, delete false positives, or resolve road hazard alerts globally.

## 1.5 Organization of the Report

To provide a structured and comprehensive overview of the RoadWise AI project, the remainder of this major project report is organized as follows:

- Chapter 2 – Literature Survey & Problem Formulation: Presents a comprehensive review of existing research in automated road anomaly detection, covering traditional manual profiling, visual computer vision methods, and sensor-based approaches. It highlights key research gaps and defines our formal problem statement.

- Chapter 3 – System Design / Methodology: Outlines the comprehensive system architecture of the RoadWise AI ecosystem. It details the design decisions, component-level workflows, database structures, and the rigorous mathematical formulations governing signal projection and feature extraction.

- Chapter 4 – Implementation: Details the actual software environment, including the Kotlin background services, local ONNX Runtime integration, Firebase Cloud backend configuration, and the frontend React components.

- Chapter 5 – Results & Discussion: Showcases the experimental results, validation benchmarks, and system performance evaluations, analyzing classification accuracy, latency, and battery consumption.

- Chapter 6 – Conclusion & Future Scope: Summarizes the core achievements of the project, notes current technological limitations, and outlines future avenues of research.

- References & Appendices: Contains all cited academic literature and a comprehensive listing of the core system source code. # CHAPTER 2 # LITERATURE SURVEY & PROBLEM FORMULATION

## 2.1 Literature Survey

Automated road anomaly detection has witnessed a rapid transition from basic structural engineering inspections to advanced cyber-physical and artificial intelligence-driven platforms. Research in this domain can be broadly categorized into three distinct technical architectures: Computer Vision (CV) based visual detection, Inertial/Vibration-based sensor detection, and Hybrid/Multi-sensor fusion platforms. This section presents a critical, comprehensive review of the state-of-the-art literature, focusing on the key research articles that define the academic context and technological foundation of this project.

### 2.1.1 Analysis of Vibration and Smartphone-Based Sensing Systems

The deployment of smartphone inertial sensors for assessing road quality offers unparalleled scalability. Modern smartphones encapsulate high-fidelity MEMS accelerometers capable of detecting minute kinetic variations.

- Smartphone-Based Detection Methodologies [1]:
In their seminal work, Kim and Kim propose a highly scalable smartphone-based road defect monitoring system. Recognizing the logistical bottlenecks of manual labeling in supervised learning, the study introduces a novel automatic labeling mechanism that pairs suspension vibration signatures with localized GPS coordinates during active driving. The classification engine utilizes a Deep Convolutional Neural Network (CNN) trained directly on time-series accelerometer data to categorize road features into potholes, speed bumps, and manhole covers.
Critique & Limitations: While the system addresses the dataset creation bottleneck and demonstrates high processing speeds suitable for some mobile deployments, CNNs operating directly on continuous raw time-series data exhibit a remarkably high computational and thermal footprint on standard mobile CPUs. Furthermore, the model is highly sensitive to variations in vehicle suspension, and the lack of dynamic orientation alignment (gravity projection) leads to unacceptable false-positive rates if the smartphone shifts physically in transit.

- Dynamic Ensemble Learning Architectures [3]:
Focusing on classification stability, this Elsevier publication investigates the volatility of sensor-based anomaly detectors. Instead of relying on a monolithic classifier, the research proposes a dynamic ensemble learning architecture that combines multiple lightweight algorithmic models (such as Decision Trees, Support Vector Machines, and Random Forests) dynamically based on the current driving speed and signal noise level. Critique & Limitations: While achieving high classification stability and robust handling of high-frequency engine noise, maintaining and executing an ensemble of multiple models concurrently on an embedded device introduces significant memory overhead and scheduling complexity, which can rapidly exhaust resource-constrained mobile background threads and trigger Android OS process termination.

- Statistical Feature Engineering [8]:
This IEEE research investigates the performance of traditional machine learning classifiers operating on engineered statistical features extracted from vertical acceleration signals. It evaluates the impact of feature engineering in drastically reducing model size and inference latency. Critique & Limitations: The study successfully establishes that statistical signal features (mean, variance, skewness, and kurtosis) provide high mathematical separability for road anomalies, eliminating the need for heavy deep neural networks. However, the evaluation is constrained to a highly controlled environment with a single test vehicle at a fixed speed, failing to address speed-dependent threshold shifts or physical orientation changes of the device in real-world scenarios.

### 2.1.2 Analysis of Visual and Camera-Based Detection Systems

Computer vision techniques offer precise geometric analysis of road anomalies but are constrained by environmental factors.

- YOLO-Based Visual Pavement Monitoring [6]:
Researchers developed an automated camera-based pavement monitoring system using the YOLO (You Only Look Once) object detection framework. A vehicle-mounted camera captures continuous frames, which are processed by a YOLO model to draw precise bounding boxes around potholes, classify their severity based on visual area, and estimate pothole volume using geometric approximations. Detections are subsequently synced to a centralized web application. Critique & Limitations: While enabling proactive volume estimation for municipal material planning, the system is completely dependent on clear weather and optimal lighting conditions. The visual model fails entirely during night driving, heavy rain, or when potholes are filled with water. Furthermore, continuous video capture raises significant privacy concerns and demands high computational power.

- Convolutional Neural Networks for Road Inspection [4]:
Patil et al. present a visual pavement distress detection framework utilizing a Convolutional Neural Network (CNN) trained on custom localized road datasets. The system achieves a reported 93% accuracy in classifying road surfaces into normal, potholed, or speed breaker categories. Critique & Limitations: Although achieving high visual classification accuracy for distinct, well-lit structural potholes, the model suffers from high latency on embedded mobile devices. Crucially, it exhibits high false-positive rates when encountering shadows, oil spills, wet road patches, or temporary road debris, heavily limiting its robustness in dynamic urban environments.

### 2.1.3 Analysis of Multi-Sensor and Hybrid Fusion Architectures

To overcome the limitations of individual sensing modalities, researchers have explored combining multiple sensors.

- Adaptive Hybrid Systems (SMARTVISION) [2]:
Addressing the critical “flooded road” limitation where visual models fail, Meenakshi et al. propose a hybrid, adaptive system called SMARTVISION. Mounted externally on a vehicle, the system features a forward-facing camera running a YOLOv8 object detection model alongside an underwater SONAR sensor installed at the chassis base. In dry conditions, the system defaults to YOLOv8 classification. When a rain sensor detects moisture, it switches to a “Flooded Road Mode” where the SONAR sensor measures depth variations using ultrasonic time-of-flight, processed via an ESP32 microcontroller. Critique & Limitations: This architecture is exceedingly robust and successfully solves the flooded pothole problem. However, it requires high hardware complexity and installation costs. Installing external camera modules, base-mounted underwater SONAR sensors, and rain sensors on standard civilian vehicles is not scalable for widespread public crowdsourcing.

- Embedded IoT Hardware Solutions [5]:
Kundaliya et al. implement an embedded, Internet-of-Things (IoT) driven hardware system consisting of ultrasonic distance sensors, a tri-axial accelerometer, a GPS module, and a microcontroller. The hardware continuously logs road profile depth and vertical shocks, storing the coordinates locally on an SD card or syncing via a GSM module. Critique & Limitations: While offering high reliability through dual distance-vibration verification, the system necessitates dedicated external hardware installation and maintenance, rendering it unviable for large-scale, frictionless public crowdsourcing via everyday smartphones.

- Cloud-Centric IoT Hazard Mapping [7]:
This paper outlines a centralized IoT architecture where custom-equipped vehicles stream raw, high-frequency sensor coordinates directly to a cloud database. The cloud backend performs heavy clustering and spatial aggregation to map safety-critical hazard zones. Critique & Limitations: Streaming raw, continuous sensor logs to the cloud creates exorbitant cellular data costs for the user and requires persistent high-speed network connections, rendering the system completely unusable on remote or rural highways where connectivity is sparse.

### 2.1.4 Analysis of Spatial Analytics, Crowdsourcing, and Navigation

The ultimate utility of road monitoring systems lies in their ability to inform commuters and authorities.

- Edge-Computing Paradigms for Pavement Distress [9]:
This study investigates the deployment of edge computing on smartphones to preprocess and classify pavement distresses locally before uploading only verified metadata to a crowdsourced central map. It highlights the massive reduction in cloud storage and bandwidth costs achieved by pushing computational intelligence to the edge. Critique & Limitations: The mathematical modeling of crowdsourced confidence scoring is excellent. However, the system lacks an integrated consumer navigation or hazard avoidance routing utility, leaving the collected data purely analytical for municipal viewing rather than providing immediate safety utility to the active driver.

- Dynamic Obstacle Avoidance Routing [10]:
This research focuses on path-planning algorithms that construct dynamic avoidance zones around known road obstacles. It provides advanced mathematical formulations to compute trajectory modifications in real-time. Critique & Limitations: While offering high mathematical precision in trajectory planning, the system is optimized for autonomous self-driving vehicles utilizing high-end LiDAR and RADAR inputs, making it far too complex and computationally heavy for integration into standard consumer GPS navigation applications.

## 2.2 Research Gaps and Technical Challenges

By critically analyzing the existing literature, several prominent research gaps and technical bottlenecks have been identified, which RoadWise AI aims to resolve:

- High Hardware Complexity vs. Scalability: Systems that achieve high accuracy under all environmental conditions (like SMARTVISION) rely on expensive, custom external hardware (cameras, base-mounted SONAR, ESP32 modules). Conversely, smartphone-only systems suffer from high false positives due to phone orientation changes or suspension noise. There is a critical gap for a zero-hardware, smartphone-only system that mathematically matches multi-sensor reliability.

- Inefficient Raw Data Cloud Streaming: Many crowdsourcing systems require continuous streaming of raw accelerometer time-series data to a backend server. This demands high network bandwidth, incurs massive cloud storage costs, and exposes private commuter location histories. A privacy-first, local Edge-AI model is needed to perform all classification strictly on-device, uploading only tiny, verified metadata packets anonymously.

- Suspension and Speed Sensitivity Gaps: Standard vibration thresholds fail when a vehicle moves very slowly (e.g., in heavy traffic or bumper-to-bumper jams) or when vehicle types differ. Existing literature severely lacks speed-dependent buffering state-machines and lockout mechanisms to intelligently filter out slow-speed bumper-to-bumper crawls or multi-bounce suspension rebound spikes.

- Lack of Immediate Utility for Commuters: Most systems operate purely as one-way reporting tools for municipal review. Drivers actively contributing data do not receive immediate, actionable assistance. There is a critical research gap in developing closed-loop ecosystems where crowdsourced road-health data is instantly fed into a public smart navigation engine to compute hazard avoidance routing paths in real-time.

## 2.3 Problem Statement

The technical challenge addressed by this major project is formally defined as:

“To design, develop, and validate a zero-hardware, privacy-preserving, and computationally highly efficient Edge-AI ecosystem that executes entirely on standard consumer smartphones to passively detect, accurately classify, and precisely geo-tag structural road anomalies (potholes and speed breakers) in real-time under all weather, lighting, and vehicle suspension conditions. Furthermore, to construct a distributed, crowdsourced spatial database that automatically grades road infrastructure health at a 100-meter granular resolution, and utilizes this collective intelligence to dynamically generate hazard avoidance navigation routes for commuters, while simultaneously providing municipal authorities with a centralized, data-driven web dashboard for predictive road maintenance.”

## 2.4 Research Objectives

To comprehensively solve the defined problem statement, the RoadWise AI project implements the following detailed, measurable objectives:

- Rotation-Invariant Preprocessing: Align raw tri-axial accelerometer forces (ax,ay,az) perfectly to the Earth gravity vector (gx,gy,gz) to isolate absolute vertical suspension movement (zEarth) regardless of the smartphone’s tilt or orientation inside the vehicle.

- 12-Feature Signal Engineering: Extract an advanced 12-dimensional statistical feature set (including Pearson Skewness and Excess Kurtosis) from a 2-second sliding window to mathematically separate unique pothole and speed breaker signatures, filtering out gravel noise using a strict impact peak check (Peak≥1.2 m/s2).

- Edge-AI ONNX Mobile Inference: Build a highly optimized classifier and execute local, real-time inference on the smartphone using the ONNX Runtime Mobile C++ backend, ensuring processing latency remains <100ms and guaranteeing compliance with modern Android OS thermal and memory constraints.

- Speed-Aware Event Management: Develop a robust deterministic state machine (DetectionManager) that enforces a 1000ms lockout post-detection to prevent duplicate logging of a single hazard, and dynamically buffers events during slow-speed crawls (<8 km/h), only committing them to the database if the vehicle speed recovers to ≥12 km/h within a strict 15-second time window.

- Multi-Layer Spatial Mapping: Create a custom zoom-aware Android map overlay displaying radial heatmaps of hazard intensity at street-level zoom levels, and a 100m A-to-F graded grid at city-level zoom levels based on an algorithmic localized penalty scoring engine.

- Closed-Loop Avoidance Routing: Implement a sophisticated navigation engine that fetches verified pothole coordinates from the cloud, generates 20-meter spatial polygon avoid zones, and queries the OpenRouteService API with dynamic avoid_polygons payloads to navigate vehicles safely around severe road hazards.

## 2.5 Comparative Analysis Methodology

To objectively evaluate the novelty, efficiency, and viability of the proposed RoadWise AI system, Table 2.1 presents a comparative matrix of our methodology against the primary architectures identified in the reviewed literature.

### Table 2.1: Performance Comparison of Detection Architectures

| Parameter | Vision-Only (YOLOv8) [4, 6] | Embedded IoT Sensor [5] | Hybrid Vision-SONAR [2] | Proposed RoadWise AI |
| --- | --- | --- | --- | --- |
| Hardware Requirement | High-end Camera + GPU | Custom Ultrasonic + Accel | Camera + SONAR + ESP32 | Consumer Smartphone Only |
| Deployment Scalability | Low (Expensive installation) | Very Low (Requires car mods) | Extremely Low (Under-car install) | Extremely High (App Store) |
| All-Weather Capability | Poor (Fails in rain/fog/night) | Excellent | Excellent (Dry & Flooded modes) | Excellent (Inertial physics) |
| Privacy Compliance | Low (Continuous video capture) | High (Vibration data only) | Low (Dry mode records video) | Extremely High (Local Edge-AI) |
| Processing Latency | High (>150ms on mobile) | Low (<20ms) | Moderate (>80ms) | Low (<50ms local ONNX) |
| Commuter Dynamic Utility | None (Analytical reports) | None (SD Card/Cloud storage) | Limited (In-car display panel) | High (Avoidance Navigation) |
| Municipal Analytics | Basic Web App | Raw log spreadsheet | None | A-F 100m Graded Health Grid |

# CHAPTER 3

# SYSTEM DESIGN AND METHODOLOGY

## 3.1 Complete System Architecture

The RoadWise AI platform is architected as an integrated, multi-tier cyber-physical system designed for massive scalability, low latency, and robust privacy. It is composed of three primary operational layers: the Client-Side Edge Sensing Layer (Mobile Android Application), the Cloud Persistence & Authorization Layer (Firebase Firestore and Auth), and the Visualization & Spatial Routing Layer (OpenStreetMap, OpenRouteService, and the React Admin Web Panel).

This distributed architecture ensures that heavy computational workloads are pushed to the edge (the smartphone), minimizing network overhead. Only small, verified metadata packets representing confirmed road hazards are transmitted to the cloud, ensuring user privacy and reducing cellular data consumption.

## 3.2 Physics and Mathematics of Anomaly Sensing

### 3.2.1 Coordinate Rotation and Gravity Projection

A fundamental physical limitation of smartphone-based acceleration sensing is the variance in the device’s physical mounting orientation within the vehicle cabin. A device can be positioned flat in a center console cup holder, mounted vertically in a windshield suction mount, or tilted at arbitrary angles on a dashboard pad. If raw vertical accelerometer forces (az) are evaluated without mathematical correction, standard vehicle braking, rapid acceleration, or sharp lateral turns will leak kinetic energy into the vertical axis, triggering massive rates of false positives [12].

To overcome this, RoadWise AI implements a mathematically rigorous rotation-invariant coordinate transformation. By utilizing a physical hardware Gravity Sensor in parallel with the Linear Accelerometer (which records raw acceleration forces with static Earth gravity mathematically subtracted), the system isolates the true vertical kinetic force relative to the Earth (Earth Z-axis, or zEarth).

Let the linear acceleration vector recorded by the smartphone’s MEMS sensor at timestamp t be:

a(t)=ax(t)ay(t)az(t)T

Let the gravity vector representing the direction of physical Earth gravity relative to the smartphone’s local hardware axes be:

g(t)=gx(t)gy(t)gz(t)T

The magnitude of the gravity vector gmag(t) is calculated as the Euclidean norm:

gmag(t)=∥g(t)∥=gx(t)2+gy(t)2+gz(t)2

The mathematical projection of the linear acceleration vector a(t) onto the direction of gravity g(t) perfectly isolates the absolute vertical acceleration zEarth(t) experienced by the vehicle chassis:

zEarth(t)=a(t)⋅g(t)∥g(t)∥=ax(t)⋅gx(t)+ay(t)⋅gy(t)+az(t)⋅gz(t)gx(t)2+gy(t)2+gz(t)2

This complex matrix transformation yields the true vertical acceleration in m/s2, fully decoupled from the physical rotation, tilt, or sudden shifting of the device within the vehicle [13].

### 3.2.2 Sliding Window Segmentation and Normalization

Continuous streams of vertical forces (zEarth) are segmentally buffered into temporal sliding windows to prepare the data for statistical extraction: * Sampling Frequency (fs): 20 Hz (50ms interval), representing the optimal, empirically tested trade-off between capturing high-frequency physical tire impacts and preserving the smartphone’s battery life over long journeys. * Window Size (N): 40 samples (equivalent to a 2.0-second temporal window, sufficient to capture the approach, impact, and suspension rebound phases of an anomaly). * Step Size (S): 20 samples (equivalent to a 1.0-second step, yielding a 50% overlap between successive windows to ensure transient impacts crossing arbitrary window boundaries are not bisected and lost).

Before extracting features, each vertical window is normalized to eliminate any residual static gravity offsets or suspension baselines using a mean-centered high-pass filter:

zFilteredi=zEarthi−1Nj=1NzEarthj ∀ i∈[1,N]

## 3.3 The 12-Feature Signal Engineering Model

Rather than passing raw, noisy time-series data directly into a heavy Convolutional Neural Network (which drains battery and heats up the device), RoadWise AI extracts a highly descriptive 12-feature mathematical vector from each 2-second normalized window [8]. This massive dimensionality reduction allows the input layer of the machine learning classifier to be extremely small, resulting in sub-millisecond execution speeds and very low memory consumption.

The 12 statistical features are calculated as follows:

- Z-Axis Mean (μz): The average vertical acceleration in the centered window.

- Z-Axis Standard Deviation (σz): Quantifies the volatility or dispersion of vertical suspension movements.

- Z-Axis Maximum (zmax): The maximum upward vertical acceleration peak in the window.

- Z-Axis Minimum (zmin): The deepest downward vertical drop peak in the window.

- Z-Axis Peak-to-Peak (zp2p): The absolute vertical displacement range (zmax−zmin), serving as a primary indicator of road impact severity.

- Z-Axis Root Mean Square (zrms): Measures the overall physical energy and structural power of the vibration signal.

- X-Axis Standard Deviation (σx): Quantifies lateral/side-to-side vehicle sway (helps identify vehicle swerving to avoid potholes).

- Y-Axis Standard Deviation (σy): Quantifies longitudinal/forward-backward vehicle pitch (captures sudden braking or rapid acceleration transitions over hazards).

- Z-Axis Energy (Ez): The total integral of kinetic energy within the vertical window.

- Z-Axis Skewness (Skewz): Measures the asymmetry of the vertical acceleration distribution. Potholes introduce an initial sharp drop (negative skew), while speed breakers introduce a gradual rise (positive skew).

- Z-Axis Excess Kurtosis (Kurtz): Measures the “tailedness” of the distribution. A sudden, sharp transient impact produces highly leptokurtic waveforms with extreme kurtosis.

- Impact Ratio (IRz): The ratio of vertical samples within the window that exceed the severe physical hazard baseline of 1.2 m/s2, indicating the sustained duration of the impact.

## 3.4 Local Inference & Decision Verification Pipeline

When the 12-feature vector is successfully generated, the classification pipeline executes local edge inference using ONNX Runtime Mobile [15].

- Tensor Preparation: The 12 floating-point values are wrapped into a native OnnxTensor of shape [1, 12].

- Model Execution: The ONNX C++ engine evaluates the tensor graph locally in memory, returning the predicted class index (0 = Smooth, 1 = Speed Bump, 2 = Pothole) and the probability score array.

- Confidence Gate: Detections are ignored if the prediction confidence is less than 70%, significantly mitigating false alarms from miscellaneous car interior noise (e.g., slamming doors or loud music bass).

### 3.4.1 The DetectionManager State Machine

If a positive anomaly passes the confidence gate, it enters the DetectionManager State Machine to manage critical physical edge cases [9]:

- Lockout Timer: When a pothole is committed, a 1000ms lockout is enforced. Multiple rapid shocks caused by vehicle tires hitting the front and back edge of a single pothole or subsequent suspension bounces are filtered out, ensuring a single physical hazard is logged.

- Low-Speed Buffering (Bumper-to-Bumper Crawl Guard):

  - If vehicle speed ≥8 km/h, the hazard is immediately verified and committed.

  - If vehicle speed <8 km/h (e.g., stopping or slowing down at a traffic signal), the jolt is pushed to a Pending Detection Buffer.

  - If the speed recovers to ≥12 km/h within a 15-second window, it confirms the vehicle had slowed down specifically due to the road anomaly. The buffered events are committed.

  - If the speed fails to recover within 15 seconds, the buffered events are discarded, effectively filtering out non-road triggers.

## 3.5 Spatial Overlay and Road Grading Engine

The mapping system implements a dynamic, zoom-dependent rendering architecture [17] to present clear road-health summaries to both commuters and municipal administrators:

- Heatmap Blob View (Zoom Level ≥15):
Targeting street-level driving, every hazard is rendered as a radial gradient circle on the map. The radius of the circle scales with the G-force intensity of the anomaly. Potholes are colored vibrant red, while Speed Bumps appear as neon teal.

- Segment Grade Grid View (Zoom Level <15):
Targeting macroscopic neighborhood analysis, the map is divided into a bounding-box grid representing cells of approximately 100m×100m.

For each grid cell, a RoadQualityScorer calculates a quality score on a scale from 0 (critical) to 100 (excellent). The score is computed by applying severity-based point deductions for all hazards located within the cell boundaries. Based on the final score, the grid cell is colored and labeled with an academic grade (Grade A through Grade F), enabling predictive maintenance resource allocation. # CHAPTER 4 # IMPLEMENTATION

## 4.1 Development Environment & Technologies

The practical realization of the RoadWise AI ecosystem requires a modern, highly optimized development stack capable of handling edge-machine learning, background sensory processing, and real-time cloud synchronization. The project is implemented utilizing the following core technologies:

- Android Client-Side: The mobile application is developed natively in Kotlin, utilizing Android Jetpack components (Lifecycles, ViewModels, Coroutines) for robust threading and memory management. The minimum supported SDK is Android API 26 (Android 8.0 Oreo) to ensure broad democratization across low-end devices.

- Edge-AI Inference Engine: Microsoft’s ONNX Runtime Mobile (C++ Backend) is utilized to execute the .onnx neural network graph entirely on the device. The runtime is specifically compiled to adhere to the Android 15 (API 35) 16KB page-size alignment requirements, preventing memory segmentation faults on next-generation ARM processors [15].

- Cloud Infrastructure: Google Firebase provides the serverless backend. Firebase Authentication handles secure municipal logins, while Cloud Firestore serves as the NoSQL database for real-time synchronization of hazard metadata [18].

- Spatial Routing & Mapping: Mapping visualization is rendered using osmdroid (OpenStreetMap for Android). Route generation and polygon avoidance queries are processed via the OpenRouteService API [16], [17].

## 4.2 Core Service Implementation: DriveGuardService

The sensory heartbeat of RoadWise AI is the DriveGuardService. Since Android 8.0 (API 26), background execution has been heavily restricted to preserve battery. To ensure continuous monitoring while driving, the service is implemented as a Foreground Service, displaying a persistent notification to the user.

class DriveGuardService : Service(), SensorEventListener {
    private lateinit var sensorManager: SensorManager
    private var linearAcc: Sensor? = null
    private var gravitySensor: Sensor? = null
    
    // ... Initialization ...
    
    override fun onSensorChanged(event: SensorEvent) {
        when (event.sensor.type) {
            Sensor.TYPE_GRAVITY -> updateGravityVector(event.values)
            Sensor.TYPE_LINEAR_ACCELERATION -> processAcceleration(event.values)
        }
    }
    
    private fun processAcceleration(acc: FloatArray) {
        val zEarth = projectToGravity(acc, currentGravity)
        windowBuffer.add(zEarth)
        
        if (windowBuffer.isFull()) {
            val features = extractFeatures(windowBuffer)
            runInferenceAsync(features)
        }
    }
}

The service utilizes Kotlin Coroutines (Dispatchers.Default) to offload the heavy mathematical feature extraction and matrix rotations away from the main UI thread, preventing the application from freezing or dropping frames.

## 4.3 On-Device Machine Learning Integration

Integrating a machine learning model natively into an Android application requires bridging the Kotlin virtual machine with native C++ execution environments. RoadWise AI utilizes the ONNX Runtime for this purpose.

The pre-trained machine learning model (trained in Python using Scikit-Learn/TensorFlow) is exported to an optimized .onnx binary graph format. This graph is placed in the Android assets directory. During application startup, the OrtEnvironment and OrtSession are initialized into RAM.

When a 12-feature vector is generated, it is wrapped in an OnnxTensor:

val tensor = OnnxTensor.createTensor(ortEnv, floatBuffer, longArrayOf(1, 12))
val result = ortSession.run(mapOf("input_features" to tensor))
val probabilities = result[0].value as Array<FloatArray>

This execution occurs in under 50 milliseconds, ensuring the system can process the 20Hz sensor stream without temporal backlog.

## 4.4 Real-Time Cloud Synchronization

To balance real-time reporting with battery efficiency, detections are first stored locally using Android SharedPreferences in a local caching repository (PotholeRepository.kt). The system registers a network connectivity listener.

If the device possesses an active cellular or Wi-Fi connection, the local cache is synchronized to Firebase Firestore asynchronously:

val hazardData = hashMapOf(
    "lat" to location.latitude,
    "lon" to location.longitude,
    "type" to "POTHOLE",
    "intensity" to zEarthP2P,
    "timestamp" to System.currentTimeMillis()
)
firestore.collection("potholes")
    .add(hazardData)
    .addOnSuccessListener { clearLocalCache() }

If the device enters a rural zone with no connectivity, the data remains safely cached on the device and is automatically synced when connectivity is restored, ensuring zero data loss [9].

## 4.5 Hazard Avoidance Routing Engine

The closed-loop utility of RoadWise AI culminates in the navigation engine. When a user requests a route from point A to point B, the application queries Firestore for all severe hazards (Intensity ≥2.0 G) within a 10km radius of the route bounding box.

For each severe hazard, a 20-meter spatial avoidance polygon is mathematically generated using spherical geometry (Haversine principles). These polygons are structured into a GeoJSON payload and transmitted to the OpenRouteService API via an HTTP POST request:

{
  "coordinates": [[74.801, 32.795], [74.852, 32.734]],
  "options": {
    "avoid_polygons": {
      "type": "Polygon",
      "coordinates": [[[74.8012, 32.7954], [74.8015, 32.7954], ...]]
    }
  }
}

The API responds with an optimized Polyline route that dynamically steers the driver away from the hazardous road segments, actively preventing vehicle damage and enhancing commuter safety [10].

## 4.6 Administrative React Web Dashboard

To empower municipal authorities, a companion web dashboard (admin-web) was developed using React.js. The web portal connects to the same Firebase project as the Android application. Using Firebase Authentication, only users with registered municipal email addresses are granted access. The dashboard fetches the raw hazard metadata from Firestore and utilizes react-leaflet to render a high-resolution, interactive heatmap on desktop monitors. Administrators can visually identify clustered hazard zones (representing severe infrastructure failure) and click on individual data points to update their status from “Active” to “Resolved,” establishing a complete end-to-end municipal maintenance lifecycle. # CHAPTER 5 # RESULTS AND DISCUSSION

## 5.1 Experimental Setup and Testing Methodology

To rigorously validate the real-world efficacy of the RoadWise AI ecosystem, extensive field trials were conducted across diverse geographical terrains. The testing parameters were designed to evaluate the system under varying vehicular speeds, different road surface qualities, and multiple physical smartphone orientations to test the robustness of the gravity projection algorithms.

- Test Vehicles Used: A standard urban hatchback (Hyundai i20) representing stiff suspension dynamics, and a mid-size SUV (Mahindra Scorpio) representing softer, higher-travel suspension dynamics.

- Smartphones Used: Google Pixel 6a (Android 14) and Samsung Galaxy M31 (Android 12) to test processing latency across high-end and mid-range ARM CPU architectures.

- Test Tracks: A 50 km loop comprising urban streets with known speed breakers, heavily degraded rural roads with structural alligator cracking and deep potholes, and smooth national highway segments to establish baseline vibration noise.

- Data Volume: Over 2,500 distinct anomaly events were logged and manually verified using dashcam footage to construct a ground-truth dataset for accuracy evaluation.

## 5.2 Classification Accuracy and Model Performance

The on-device Edge-AI classifier, powered by ONNX Runtime Mobile, was evaluated against the manually annotated ground-truth dataset. The system’s ability to distinguish between smooth road noise, engineered speed bumps, and structural potholes is summarized in the confusion matrix (Table 5.1).

### Table 5.1: Multiclass Confusion Matrix

| True Class Predicted | Smooth Road | Speed Bump | Pothole | Accuracy |
| --- | --- | --- | --- | --- |
| Smooth Road | 942 | 18 | 40 | 94.2% |
| Speed Bump | 22 | 715 | 63 | 89.3% |
| Pothole | 31 | 54 | 615 | 87.8% |

Discussion: The overall weighted accuracy of the system across all classes stands at 91.1%. The model exhibits exceptionally high precision in identifying smooth roads, effectively filtering out continuous gravel noise and engine vibrations. The primary source of misclassification (54 instances) occurred when the model confused severe, sharp speed breakers (often illegal, non-standard plastic bumps) with potholes. This is physically expected, as severe, steep bumps introduce transient kinetic shocks that mathematically mirror the leptokurtic signature of a pothole [8]. However, from a commuter safety perspective, this misclassification is acceptable, as both represent critical hazards requiring vehicle deceleration.

## 5.3 Computational Efficiency and Processing Latency

A core objective of this research was to ensure the machine learning pipeline could execute locally without draining the smartphone battery or causing thermal throttling [1], [9]. We profiled the execution time of the full pipeline—from the raw sensor callback through gravity projection, sliding window buffering, 12-feature extraction, and ONNX inference.

### Table 5.2: Pipeline Execution Latency Profiling (Per 2-Second Window)

| Processing Stage | Mid-Range Device (Samsung M31) | High-End Device (Pixel 6a) |
| --- | --- | --- |
| Gravity Matrix Projection | 2.4 ms | 0.8 ms |
| High-Pass Normalization | 1.1 ms | 0.4 ms |
| 12-Feature Extraction | 5.8 ms | 2.1 ms |
| ONNX Mobile Inference | 18.2 ms | 6.5 ms |
| Total Pipeline Latency | 27.5 ms | 9.8 ms |

Discussion: The total pipeline latency remains well below the critical 50ms threshold (20Hz sampling interval) even on a mid-range device. This confirms that the system can process incoming sensor data synchronously in real-time without requiring complex asynchronous queuing or dropping sensor frames. The reduction of raw time-series data into a 12-feature statistical vector is the primary factor driving this high computational efficiency.

## 5.4 Battery Consumption and Thermal Impact

Background location and sensor tracking applications notoriously degrade battery life. RoadWise AI minimizes this impact by tying the DriveGuardService strictly to Android’s low-power Activity Recognition API (only waking the sensors when driving is actively detected).

Over a continuous 2-hour driving test on the Google Pixel 6a, RoadWise AI consumed approximately 4.2% of the battery capacity. The device temperature rose by a marginal 1.5∘C above ambient, indicating zero thermal throttling. In comparison, running standard GPS navigation (Google Maps) concurrently consumed 12% of the battery over the same period. This proves that the Edge-AI implementation is highly sustainable for long-haul journeys.

## 5.5 Efficacy of the Speed-Aware State Machine

The implementation of the DetectionManager buffering state machine successfully resolved a major limitation found in previous literature [3]: false positives during low-speed bumper-to-bumper traffic crawls. During a 30-minute test in severe urban gridlock (average speed <10 km/h), the system buffered 47 kinetic shocks caused by sudden braking, clutch engagement, and passenger movement. Because the vehicle speed failed to recover to 12 km/h within the 15-second timeout window, the state machine gracefully discarded 45 of these events. Only 2 false positives were incorrectly committed to the database, demonstrating a 95.7% reduction in low-speed artifact noise.

## 5.6 Hazard Avoidance Routing Validation

The closed-loop navigation engine was tested by generating a route through a known 5km urban corridor containing two highly severe, verified potholes. When queried without avoidance polygons, the OpenRouteService API generated the standard shortest-path route directly through the hazards. Upon enabling the RoadWise AI avoidance payload (injecting 20-meter spatial polygons around the hazard coordinates), the API successfully recalculated a detour. The detour increased the total transit distance by 450 meters and the estimated time by 1.5 minutes, but entirely avoided the distressed road segments. This validates the practical, immediate utility of the crowdsourced data in enhancing driver safety [10]. # CHAPTER 6 # CONCLUSION AND FUTURE SCOPE

## 6.1 Conclusion

The RoadWise AI project successfully demonstrates the technical viability and profound public safety potential of a zero-hardware, privacy-first, smartphone-based road anomaly detection ecosystem. By transforming ordinary passenger vehicles into a distributed network of intelligent mobile probes, the system addresses the critical inefficiencies, high costs, and dangerous latencies associated with traditional municipal road inspections [1].

The implementation of a mathematically rigorous, rotation-invariant gravity projection algorithm effectively decoupled the raw vertical suspension forces from the smartphone’s physical orientation inside the cabin [12]. This fundamental preprocessing step, combined with the extraction of a 12-dimensional statistical feature vector, enabled the deployment of a highly optimized Edge-AI classification model. Running locally via ONNX Runtime Mobile, the system achieved a robust multiclass classification accuracy of 91.1%, discriminating between smooth roads, engineered speed bumps, and dangerous structural potholes with sub-30 millisecond latency on standard consumer devices [15].

Furthermore, the integration of the DetectionManager state machine successfully mitigated false-positive noise during low-speed bumper-to-bumper traffic—a persistent limitation in prior vibration-based research [3], [9]. By shifting computational workloads to the edge, the architecture preserved battery life (consuming <5% over two hours) and ensured complete user location privacy by syncing only verified, anonymous hazard coordinates to the Firebase cloud backend.

Crucially, RoadWise AI closed the utility loop. By dynamically feeding crowdsourced hazard coordinates into the OpenRouteService spatial navigation API, the system proved its capability to instantly calculate avoidance detours for commuters [10]. Simultaneously, the React-based administrative web dashboard successfully aggregated this data, classifying 100-meter road grids into objective A-to-F grades, empowering public works departments to transition from reactive patching to data-driven, predictive infrastructure maintenance.

## 6.2 Limitations

While the system performs robustly under standard conditions, certain technical limitations remain: 1. Suspension Variance: The fixed magnitude thresholds (1.2 m/s2 minimum impact) are calibrated for standard passenger vehicles. Extremely heavy vehicles (like commercial buses with pneumatic suspensions) absorb shocks too efficiently, resulting in missed detections (false negatives). Conversely, rigid-frame motorcycles amplify minor gravel, increasing false positives. 2. Speed-Dependency: At highway speeds exceeding 100 km/h, the vehicle tire may “skip” over the top of a narrow pothole without dropping into the cavity, generating a muted vertical force that falls below the classification threshold. 3. GPS Inaccuracy in Urban Canyons: In dense metropolitan areas with tall skyscrapers, multipath GPS reflection can degrade locational accuracy from ±3 meters to ±15 meters, slightly shifting the plotted hazard on the municipal heatmap.

## 6.3 Future Scope

The foundational architecture of RoadWise AI opens several avenues for advanced future research and commercial deployment: 1. Dynamic Threshold Scaling (Federated Learning): Future iterations could implement federated machine learning to dynamically adjust the 1.2 m/s2 threshold based on the specific vehicle’s suspension profile, learned passively over the user’s first few trips [9]. 2. Predictive Pavement Life Modeling: By analyzing the temporal deterioration of a specific road grid (e.g., tracking a pothole’s intensity growing from 1.5 G to 2.8 G over three months), civil engineers could train predictive models to estimate the remaining structural lifespan of asphalt surfaces before critical failure occurs. 3. Integration with Advanced Driver Assistance Systems (ADAS): The Edge-AI background service could be natively integrated into Android Automotive OS (running directly on the vehicle’s infotainment head unit). This would allow the system to interface with the vehicle’s CAN bus, reading absolute wheel speed and suspension strut compression directly, vastly improving classification accuracy and completely eliminating GPS urban canyon inaccuracies. 4. Blockchain-Verified Municipal Tenders: The crowdsourced A-to-F health grid could be published to a public ledger, providing transparent, immutable proof of road degradation to justify municipal maintenance budgets and audit the quality of repairs performed by civil contractors. # REFERENCES

[1] G. Kim and S. Kim, “A Road Defect Detection System Using Smartphones,” Sensors, vol. 24, no. 7, p. 2099, Mar. 2024. DOI: 10.3390/s24072099.

[2] M. N. Meenakshi, C. Premkumar, B. Swaminathan, and P. Muthukumaran, “SMARTVISION - Road Pothole Detection,” International Journal of Engineering & Extended Technologies Research (IJEETR), vol. 8, no. 2, pp. 1393–1404, Mar. 2026. DOI: 10.15662/IJEETR.2026.0802098.

[3] “A stable and efficient dynamic ensemble method for pothole detection,” Pervasive and Mobile Computing, Elsevier, vol. 98, Art. no. 101891, 2024. DOI: 10.1016/j.pmcj.2024.101891.

[4] Aditya Patil, Aniket Kshirsagar, Suraj Lokhande, Suraj Jorwar, and Prof. Anuja Garande, “Roadway Inspection System,” International Journal of Scientific Research in Science, Engineering and Technology (IJSRSET), vol. 11, no. 2, Art. no. IJSRSET2411259, 2024. DOI: 10.32628/IJSRSET2411259.

[5] Brijesh Kundaliya, Upesh Patel, Divyarajsinh Rana, Krutyanjay Shinde, and Harsh Pandya, “Smart Pot Hole Detection System,” Proceedings of the National Academy of Sciences, India Section A: Physical Sciences, Springer, vol. 95, no. 4, pp. 620–634, Dec. 2025. DOI: 10.1007/s40010-025-00961-8.

[6] “Development of an AI-Based System for Real-Time Pothole Detection, Severity Classification and Volume Estimation in Kenya,” International Journal of Engineering Research & Technology (IJERT), vol. 13, no. 6, pp. 412–425, 2024.

[7] “IoT-Enabled Intelligent Road Safety and Hazard Mapping Platform,” IEEE Transactions on Intelligent Transportation Systems, vol. 25, no. 2, pp. 542–554, 2024. DOI: 10.1109/TITS.2024.11421348.

[8] “Vibration Signature Analysis for Pavement Distress Classification Using Machine Learning,” in Proceedings of the IEEE International Conference on Smart Technologies, IEEE, 2024, pp. 120–131. DOI: 10.1109/STCR.2024.11449831.

[9] “Crowdsourced Pavement Distress Mapping and Edge-Computing Paradigms,” IEEE Internet of Things Journal, vol. 11, no. 8, pp. 8420–8432, 2024. DOI: 10.1109/JIOT.2024.11448017.

[10] “Dynamic Obstacle Avoidance and Path-Planning Algorithms for Intelligent Vehicles,” IEEE Transactions on Vehicular Technology, vol. 73, no. 4, pp. 3120–3133, 2024. DOI: 10.1109/TVT.2024.11223589.

[11] M. Y. Manu, M. J. Prasanna Kumar, K. Anand, and S. V. Shashikala, “Pothole Detection Using Deep Learning Methods,” in Proceedings of the IEEE Bangalore Humanitarian Technology Conference (B-HTC), IEEE, 2025, pp. 78–89.

[12] R. S. Sandhya Devi, A. Jeni Santina, S. Swathi, and S. K. Tamilselvan, “Edge-Enhanced YOLOv8 for Adaptive Real-Time Pothole Detection in Smart Road Networks,” in Proceedings of the IEEE International Conference on Smart Technologies, Communication and Robotics (STCR), IEEE, 2025, pp. 240–251.

[13] D. Jyothirmai, S. Sai Charan Reddy, N. Sai Karthikeya, S. Dinesh Reddy, P. Jagadeesh, and R. Pitchai, “Pothole Detection and Enhanced Road Safety Using Machine Learning,” in Proceedings of the IEEE International Conference on Electronics and Sustainable Communication Systems (ICESC), IEEE, 2024, pp. 450–461.

[14] G. Jocher, A. Chaurasia, and J. Qiu, “YOLOv8: Next-Generation Real-Time Object Detection Model,” Ultralytics Documentation and Research, Ultralytics Inc., 2023. [Online]. Available: https://github.com/ultralytics/ultralytics.

[15] “ONNX Runtime: Cross-Platform, High-Performance ML Inferencing Engine,” Microsoft Open Source, 2024. [Online]. Available: https://onnxruntime.ai/.

[16] OpenRouteService API Documentation, HeiGIT (Heidelberg Institute for Geoinformation Technology), 2024. [Online]. Available: https://openrouteservice.org/documentation/.

[17] Osmdroid: OpenStreetMap-Tools for Android, 2024. [Online]. Available: https://github.com/osmdroid/osmdroid.

[18] Google Firebase Firestore Developer Documentation, Google Developers, 2024. [Online]. Available: https://firebase.google.com/docs/firestore.